DIGITAL LOGIC DESIGN PROJECT

4-Bit Password Lock System

Logic Design Report

—  Complete Design Documentation  —

| Project | 4-Bit Password Lock System |
| --- | --- |
| Subject | Digital Logic Design |
| Password | 1011 (binary) = 11 (decimal) |
| Core Gates | XNOR x4 + AND (4-input) + NOT |
| Outputs | GRANT (Access Granted) / DENY (Access Denied) |

Prepared: May 2026

# 1. System Overview

The 4-Bit Password Lock System is a combinational digital circuit that compares a 4-bit user-entered code against a pre-stored 4-bit password. If every bit matches, access is granted; otherwise, access is denied. The decision is made instantly using pure combinational logic, with no clock or memory required beyond the hardwired password constants.

Key Concept This is a comparator circuit, not a multiplexer (MUX). A MUX selects and routes data using select lines. A comparator checks equality and outputs a 1-bit yes/no result. The closest real IC is the 74HC85 4-bit magnitude comparator.

## 1.1  How It Works (Step by Step)

- User enters a 4-bit code via switches or a keypad (I3 I2 I1 I0).

- Each input bit is compared to the corresponding stored password bit using an XNOR gate.

- An XNOR gate outputs 1 when both inputs are equal (0=0 or 1=1), and 0 when they differ.

- All four XNOR outputs feed into a 4-input AND gate.

- The AND gate outputs 1 only when all four bits matched (all XNOR outputs are 1).

- The AND output becomes GRANT. Its complement (via NOT gate) becomes DENY.

- Pressing RESET clears all inputs back to 0000.

Why XNOR + AND? XNOR is the equality gate: output is HIGH only when both inputs are the same. Four XNORs check all four bit-pairs simultaneously. The AND gate then enforces the requirement that ALL four must match — one wrong bit makes the AND output go LOW.

# 2. Inputs & Outputs

## 2.1  System Inputs

| Signal | Bit | Type | Description |
| --- | --- | --- | --- |
| I3 | MSB (Bit 3) | User input | Most significant bit of entered code |
| I2 | Bit 2 | User input | Second bit of entered code |
| I1 | Bit 1 | User input | Third bit of entered code |
| I0 | LSB (Bit 0) | User input | Least significant bit of entered code |
| RST | — | Control | Reset — clears all input bits to 0 |

## 2.2  Stored Password Constants

The password is hardwired into the circuit as fixed logic constants (tied to VCC for logic 1 or GND for logic 0). It cannot change at runtime.

| P3 (MSB) | P2 | P1 | P0 (LSB) |
| --- | --- | --- | --- |
| 1 | 0 | 1 | 1 |

Binary: 1011   |   Decimal: 11   |   Hex: 0xB

## 2.3  System Outputs

| Signal | Active when | Logic Level | Drives |
| --- | --- | --- | --- |
| GRANT | All 4 bits match password | HIGH (1) | Green LED, relay, door unlock |
| DENY | Any bit differs from password | HIGH (1) | Red LED, buzzer, alarm |

Important GRANT and DENY are always complementary: when GRANT = 1, DENY = 0, and vice versa. They can never both be 1 or both be 0 simultaneously.

# 3. Boolean Logic

## 3.1  XNOR Gate (Bit Comparator)

An XNOR gate is an XOR gate with an inverted output. It produces logic 1 when both inputs are equal:

| A | B | A XNOR B |
| --- | --- | --- |
| 0 | 0 | 1 (equal) |
| 0 | 1 | 0 (different) |
| 1 | 0 | 0 (different) |
| 1 | 1 | 1 (equal) |

## 3.2  Boolean Expressions

Individual bit match signals:

M3 = I3 XNOR P3 = I3•P3 + I3̅•P3̅

M2 = I2 XNOR P2 = I2•P2 + I2̅•P2̅

M1 = I1 XNOR P1 = I1•P1 + I1̅•P1̅

M0 = I0 XNOR P0 = I0•P0 + I0̅•P0̅

Combined match (AND gate):

MATCH = M3 • M2 • M1 • M0

Final outputs:

GRANT = MATCH

DENY  = NOT MATCH = MATCH̅

Expanded (with password 1011 substituted):

GRANT = (I3∕1) • (I2∕0) • (I1∕1) • (I0∕1)

= I3 • I2̅ • I1 • I0

Simplified form Because the password bits are constants, each XNOR simplifies: when Pn=1, the XNOR output equals In directly. When Pn=0, the XNOR output equals NOT(In). So with password 1011: GRANT = I3 AND (NOT I2) AND I1 AND I0.

# 4. Truth Table

The system has 4 input bits (I3 I2 I1 I0), giving 2⁴ = 16 possible input combinations. Only one combination (1011) produces GRANT = 1.

The highlighted row is the correct password.

| I3 | I2 | I1 | I0 | Decimal | M3 | M2 | M1 | M0 | GRANT | DENY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 0 | 2 | 0 | 1 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 | 3 | 0 | 1 | 1 | 1 | 0 | 1 |
| 0 | 1 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 5 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 | 6 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 7 | 0 | 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 0 | 8 | 1 | 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 9 | 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 10 | 1 | 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 11 | 1 | 1 | 1 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 12 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 13 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 | 14 | 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 15 | 1 | 0 | 1 | 1 | 0 | 1 |

M3–M0 = individual XNOR bit-match outputs. All must be 1 for GRANT to go HIGH.

# 5. Circuit Design

## 5.1  Gate-Level Architecture

The circuit consists of three stages connected in a pipeline from left to right:

| Stage | Component | Function |
| --- | --- | --- |
| 1 | 4 x XNOR gates | Each compares one entered bit to one password bit. Outputs 1 on equality. |
| 2 | 1 x 4-input AND gate | Combines all four XNOR outputs. Outputs 1 only when all four are 1. |
| 3 | 1 x NOT gate | Inverts AND output to produce DENY. GRANT and DENY are always opposite. |

## 5.2  Signal Flow

- Input bits I3–I0 (blue, solid wires) enter from the left.

- Password constants P3–P0 (amber, dashed wires) are hardwired into the XNOR inputs.

- XNOR outputs (green wires) carry the per-bit match signals M3–M0.

- The AND gate merges all four match signals into a single MATCH output.

- MATCH feeds directly to GRANT, and via the NOT gate to DENY.

## 5.3  Real-World Implementation

This circuit can be built using off-the-shelf 74HC-series logic ICs:

| IC Part Number | Function | Notes |
| --- | --- | --- |
| 74HC266 | Quad 2-input XNOR | One IC provides all four XNOR gates needed |
| 74HC21 | Dual 4-input AND gate | One IC provides the 4-input AND gate |
| 74HC04 | Hex inverter (NOT gates) | One gate used for DENY output |
| 74HC85 (alternative) | 4-bit magnitude comparator | Single IC replaces the entire XNOR + AND stage |

# 6. How the System Decides

This section explains the decision-making process in plain language, step by step.

## 6.1  The Comparison Process

| Step | Action | What happens inside | Result |
| --- | --- | --- | --- |
| 1 | Enter code | User sets I3 I2 I1 I0 via switches or keypad | 4-bit binary number available at XNOR inputs |
| 2 | Bit comparison | Each XNOR gate compares one input bit to one password bit | 4 individual match signals M3, M2, M1, M0 |
| 3 | Combine results | AND gate checks if ALL four match signals are HIGH | Single MATCH bit (1 = all matched, 0 = at least one differs) |
| 4 | Generate outputs | MATCH goes to GRANT; NOT MATCH goes to DENY | Exactly one of GRANT, DENY is HIGH at all times |
| 5 | Reset (optional) | RST signal clears all inputs back to 0000 | System returns to default DENY state |

## 6.2  Worked Example

Password = 1011. User enters 1001.

| Bit position | Input (I) | Password (P) | XNOR output | Verdict |
| --- | --- | --- | --- | --- |
| I3 / P3 | 1 | 1 | 1 | Match |
| I2 / P2 | 0 | 0 | 1 | Match |
| I1 / P1 | 0 | 1 | 0 | MISMATCH -- bit is wrong! |
| I0 / P0 | 1 | 1 | 1 | Match |

Result: ACCESS DENIED AND gate sees inputs: 1, 1, 0, 1 → output = 0 (because one input is 0). GRANT = 0, DENY = 1. Even though 3 out of 4 bits are correct, the single mismatch at I1 is enough to deny access.

## 6.3  Summary of Design Decisions

- XNOR was chosen over XOR because XNOR outputs HIGH on equality, making the logic more natural for a comparator.

- A 4-input AND gate enforces strict all-or-nothing matching — partial matches are always rejected.

- DENY is derived from GRANT via a NOT gate rather than independent logic, guaranteeing they are always complementary.

- The password is hardwired (not stored in RAM) for simplicity and tamper resistance in this design.

- RST is included so the circuit can be cleanly reset without power-cycling.

Extension Ideas Lockout after 3 wrong attempts (requires a counter + flip-flops). 7-segment display showing bit positions. Configurable password stored in non-volatile memory (EEPROM). Keypad encoder IC to generate the 4-bit input automatically.